EXPERIMENT NO. 2 (Group A)

● Aim: Study of different operating systems for Raspberry-Pi/Beagle

board/Arduino. Under standing the process of OS installation.

● Outcome: To study operating systems for platforms such as Raspberry-Pi/

Beagle board/Arduino.

● Hardware Requirement: Raspberry-Pi

● Software Requirement: Raspbian OS

● Theory:

1) Raspberry-Pi: - The Pi can run the official Raspbian OS, Ubuntu Mate, Snappy

Ubuntu Core, the Kodi- based media centers OSMC and LibreElec, the non-Linux

based Risc OS (one for fans of 1990s Acorn computers). It can also run Windows

10 IoT Core, which is very different to the desktop version of Windows, as

mentioned below.

● OS which install on Raspberry-Pi: Raspbian, Ubuntu MATE, Snappy Ubuntu,

Pidora, Linutop, SARPi, Arch Linux ARM, Gentoo Linux, etc.

⮚ How to install Raspbian on Raspberry-Pi:

⮚ Step 1: Download Raspbian

⮚ Step 2: Unzip the file. The Raspbian disc image is compressed, so you’ll need to

unzip it. The file uses the ZIP64 format, so depending on how current your built-in

utilities are, you need to use certain programs to unzip it.

⮚ Step 3: Write the disc image to your microSD card. Next, pop your microSD card

into your computer and write the disc image to it. The process of actually writing

the image will be slightly different across these programs, but it’s pretty

self-explanatory no matter what you’re using. Each of these programs will have you

select the destination (make sure you’ve picked your microSD card!) and the disc

image (the unzipped Raspbian file). Choose, double-check, and then hit the button

to write.

⮚ Step 4: Put the microSD card in your Pi and boot up. Once the disc image has been

written to the microSD card, you’re ready to go! Put that sucker into your

Raspberry Pi, plug in the peripherals and power source, and enjoy. The current

edition to Raspbian will boot directly to the desktop. Your default credentials are

username pi and password raspberry.

2) BeagleBone Black: - The BeagleBone Black includes a 2GB or 4GB on-board eMMC

flash memory chip. It comes with the Debian distribution factory pre-installed. You

can flash new operating systems including Angstrom, Ubuntu, Android, and others.

1. Os which install on BeagleBone Black: Angstrom, Android, Debian, Fedora,

Buildroot, Gentoo, Nerves Erlang/OTP, Sabayon, Ubuntu, Yocto, MINIX 3

⮚ How to install Debian on BeagleBone Black:

⮚ Step 1: Download Debian img.xz file.

⮚ Step 2: Unzip the file.

⮚ Step 3: Insert your MicroSD (uSD) card into the proper slot. Most uSD cards come

with a full-sized SD card that is really just an adapter. If this is what you have then

insert the uSD into the adapter, then into your card reader.

⮚ Step 4: Now open Win32 Disk imager, click the blue folder icon, navigate to the

debian img location, and double click the file. Now click Write and let the process

complete. Depending on your processor and available RAM it should be done in

around 5 minutes.

⮚ Step 5: Alright, once that's done, you'll get a notification pop-up. Now we're ready

to get going. Remove the SD adapter from the card slot, remove the uSD card from

the adapter. With the USB cable disconnected insert the uSD into the BBB.

⮚ Step 6: Now, this next part is pretty straight forward. Plug the USB cable in and

wait some more. If everything is going right you will notice that the four (4) leds

just above the USB cable are doing the KIT impression. This could take up to 45

minutes; I just did it again in around 5 minutes. Your mileage will vary. Go back

and surf reddit some more.

⮚ Step 7: If you are not seeing the leds swing back and forth you will need to unplug

the USB cable, press and hold down the user button above the uSD card slot (next

to the 2 little 10 pin ICs) then plug in the USB cable. Release the button and wait.

You should see the LEDs swinging back and forth after a few seconds. Once this

happens it's waiting time. When all 4 LEDs next to the USB slot stay lit at the same

time the flash process has been completed.

⮚ Step 8: Remove the uSD card and reboot your BBB. You can reboot the BBB by

removing and reconnecting the USB cable, or hitting the reset button above the

USB cable near the edge of the board.

⮚ Step 9: Now using putty, or your SSH flavor of choice, connect to the BBB using

the IP address 192.168.7.2. You'll be prompted for a username. Type root and

press Enter. By default, there is no root password. I recommend changing this

ASAP if you plan on putting your BBB on the network. To do this type password,

hit enter, then enter your desired password. You will be prompted to enter it

again to verify.

3) Arduino: - The Arduino itself has no real operating system. You develop code for

the Arduino using the Arduino IDE which you can download from Arduino - Home.

Versions are available for Windows, Mac and Linux. The Arduino is a constrained

microcontroller. Arduino consists of both a physical programmable circuit board

(often referred to as a microcontroller) and a piece of software, or IDE (Integrated

Development Environment) that runs on your computer, used to write and upload

computer code to the physical board. You are literally writing the "firmware" when

you write the code and upload it. It's both good and its bad.

------------------------------------------------------------------------------------------------------------------------

Conclusion: -